Product Specifications

| Industrial Design | ABS: Glossy and rubber finish |  | H: 172mm ; D: 71mm ; W: 475g |
| --- | --- | --- | --- |
| Air Quality Sensing | Fine Particles | - Sensing technology: light scattering, Low latency detection - Factory Calibration, on the fly signal processing - Sensibility: particulate size 0.03 µm to 2.5 µm (PM 2.5) - Range 0 mg/m3 to 1.4 mg/m³ ; Precision ±12% |  |
|  | Total VOC | - MOS sensor tech. auto motive industry grade. - High reliability and stability - Low latency detection | - Formaldehyde - Iso-Butane - Toluene - Methane - Ammonia - Benzene - Etc. |
|  | Carbon Monoxide* | *Total VOC sensor highly sensitive to CO | tVOC range: 100 to 1000 ppb |
|  | Carbone Dioxide** | **Signal processing converts tVOC levels to CO2 equivalents | Detection range: 400 to 6000 ppm |
|  | Temperature | Range ; Accuracy | -40 to +125°C; ±0.4°C |
|  | Humidity | Range ; Accuracy | 0 to 100% ; RH ±4% |
| Connectivity | Wi-Fi 802.11 B / G / N ; Security: Open / WEP / WPA / WPA2 Personal |  | Store data every 5 min to cloud On demand instant measurements |
| User interactions | Gesture (turn upside down, tap, etc.) |  | iOS (iPhone/iPad) ; Android Colored LED light output |
| Power | Not detachable USB cable (1.3 meters) |  | AC-DC 5V 0.5A USB power adaptor |